What is CP/M?

CP/M, or Control Program for Micro Computing, is a little-used mode for the Commodore 128.  I think the reason is that most people don’t know how to set it up efficiently, so they boot up their system disk once, and then forget about it.

The thing is, CP/M mode opens your Commodore 128 up to thousands of programs written from the mid-70s to today.  CP/M, for the most part, is platform independent, so a program written for a Kaypro or Osborne, will work fine on your Commodore 128.  The exception to this rule is if a program takes advantage of a particular aspect of a machine, say a graphics mode, that doesn’t exist on another computer, it may not work.  Or, for example, a machine has a limitation that the Commodore 128 doesn’t have, say a 64 column screen, then all of the features of the 128 might not be used.  Either way, the majority of CP/M programs work without issue.

CP/M is generally a text-mode only operating system.  It was developed through the mid-1980s, when it was more important to display text for business reasons than it was to have a fancy bit-mapped graphics screen.  Keep this in mind when trying out programs – it’s just about guaranteed that the program will require a lot of typing.

What’s needed for a Commodore 128 CP/M Environment

You know the old saying, just because you CAN do something doesn’t mean you should.  You CAN use a single 1571 to run your environment, but you shouldn’t.  This quickly will become a painful lesson in disk swapping.  So what do you need?

The Minimum System:

The minimum system to run CP/M properly is a Commodore 128, 80 column monitor, and 1571 disk drive.  A 1541 will not work as it is unable to read the CP/M format.  You can technically get away with a 40 column screen, however, most programs are expecting a 64 column screen or higher.    You will spend a lot of time disk swapping with this setup.

The Usable System:

The usable system would consist of a minimum of 2 disk drives that can read CP/M disks, 2 1571 disk drives, and an 80 column monitor.  One drive would hold your system files, while the other would be the programs that you would like to use.

The Recommended System:

The recommended system to get to have a Commodore 128, a 1571 for reading program disks, a 1571 or 1581 for storing your system files, an 80 column screen, and a REU of 128K or more.  CP/M will automatically mount your REU as drive M.  You can store all of your system files into this RAM drive to prevent disk swapping.

The Ultimate System:

An ultimate system would include all of the above and add in a CMD HD or compatible hard drive.   CMD’s HD-DOS natively supports CP/M partitions, and includes a special format for it.  This allows you to set up a partition for games, a partition for applications, etc.

Setting Up Your Environment

There are many CP/M images floating around the internet, but for this document, we’re going to use the ones that are attached here:

The first disk is a bootable CP/M environment that contains utilities to get your environment going.  It is *not* a full CP/M install.   Therefore, all 3 disks will need to be made.  There are some modifications on these images, so please use these CP/M images and not other ones found on the internet for this tutorial.

Note: We are going to assume that you’re using at least the ‘usable system’ configuration for CP/M.  This means we’re going to assume you have dual 1571 disk drives.

Please note, these disks are all in D64 format.  This is NOT an ideal format for CP/M because CP/M can easily use the double-sided mode of the 1571.  However, most people have a 1541 hooked up for making disk images, so this is the format I distributed them on.

Boot off the first disk named ‘CPM Side 1’.   CP/M expects the boot drive to be drive #8, so place your CPM Side 1 disk into your 1571 drive, drive #8 and turn on your 128 in 80 column mode.  If the drive does not boot automatically, you can type BOOT + <RETURN> from your BASIC prompt.

After a few minutes of drive churning, it should look like this:

A:\

MS-DOS was modelled after CP/M, so if you are familiar with MS-DOS, then CP/M should be familiar as well.

This setup guide will not go into each and every CP/M command but will cover a few of them.  For more information on CP/M’s commands, you can find the manual attached here:

Just like MS-DOS, you’ll notice your main boot drive is A.  CP/M for the Commodore 128 sets up its drives the following way:

Drive #8 – Drive A

Drive #9 – Drive B

Drive #10 – Drive C

Drive #11 – Drive D

REU – Drive M

We created our boot disk as a single-sided disk and this format is very limited on space.  The first thing we want to do is to create a double-sided disk and copy both the CPM Side 1 Disk, and the CPM Side 2 disk to it.

To do so, we’re going to need to format a double-sided disk to copy the files to.

From the A: prompt, type A:format.

Using the TOP arrows on the keyboard (not the cursor keys), arrow down to the “Select Different Drive” prompt.  We do NOT want to format your boot disk.  You will then be prompted to select your destination drive, Drive A B C or D.  Refer to the chart above to understand what disk you are formatting.  In our example, I am formatting drive #11, so I am selecting drive D.

Ensure you select the format as Commodore 128 Double Sided if it is a 1571 disk.

When you are ready to go, insert your disk and type “$”.

After some churning, your disk will be formatted. When it asks you if you want to format another disk, select ‘N’. You will then be placed back to the A: prompt.

We are now going to copy the files from your A: drive, to your newly formatted disk.  In my example, my newly formatted disk is drive D.

We’re going to use PIP, which is the CP/M for copying files, to copy files to our new double-sided disk.

Type PIP d:=a:*.*

This will copy all the files from drive A to drive D. Replace D: with whatever your destination disk drive is.

Place the CPM Side 2 disk into your A: drive, and follow the same steps as before.

Note: If you get a note such as PIP? …when you typed in PIP, this means CP/M was unable to find the command. CP/M uses a path-based system to find files.  It has a default search path of the current drive.  In other words, if you’re on drive A, and the file is on drive D, then CP/M will be unable to find it.  You will need to specify to CP/M where the file is.  D:PIP will tell CP/M that you’re looking for PIP on drive D.  You can setup the path for your needs.  We’ll go into this more later.

Congrats!  You’ve created your first system disk!

This has everything you need to boot into CP/M and use it.  However, you’ll find a lot of it lacking.  Just like other OSes, coders have created utilities that will make your life easier.  I’ve included some of these utilities on the CPM Utility Disk.   On a dual drive system, you’ll have to put the utility disk into your second drive, launch them, and then swap out disks to the disk you’re wanting to manipulate.  This alone is a lot of disk swapping, even on a dual-1571 system.

Before we go into how to make this better, let’s go over the utilities included on the utility disk:

JUGG.COM – Juggler 128.  Allows you read CP/M disks formatted by other systems.  Highly useful.

MSDOS.COM – Allows you to read/write MS-DOS formatted disks on your Commodore 128.

RDCBM.COM – Read Commodore formatted disks in CP/M mode.  You can copy to/from the Commodore environment via this command.

SPORT.COM – Allows you to send Commodore DOS commands from CP/M mode.  Useful when using CMD Hard Drives.

SUPERDIR.COM – Gives you more detailed directory information.

SWP.COM – SWEEP, a program that allows you to select files and copy them in batches.  Much easier than PIP.

ZDE.COM – Text editor.  Useful for creating text files easily.

PMARC.COM – Unarchive ARC files

MBASIC.COM – Microsoft BASIC for CP/M.  Use for running .BAS files.

PMEXT.COM – Unarchive PMA files.

Making CP/M Easier

If you’re using CP/M on a regular basis, it is highly recommended (hence why it’s called the recommended system ) to use at least a 1581 for either your system drive (A:) or your utility disk.  Having all of the files needed in a single place will prevent disk swapping.

Using an REU is even better if you have one.  CP/M automatically configures your REU as the M: drive.  You can copy all of your needed files to the M: drive so that they are available from any drive prompt.  To do so, you’ll need to change your system path search order.  Remember we went over briefly that CP/M uses a search path to find files?  By default, CP/M does not look into the M: drive.  So let’s tell CP/M about it.

From the A: prompt, type ‘setdef’.  You will be shown what CP/M is looking for, and in what order:

Drive Search Path:

1st Drive – A:

This means it’s only looking in the A: drive for files.  Let’s tell it to search more drives:

Setdef m:,a:,*

This tells CP/M to look at drive M: first, then drive A:, then whatever the current drive is.

Now if you type a command, and the command is stored in your REU (M:) drive, it will be found first and launched.

To copy a file into your REU (M:) drive, you will need to use the PIP command.  We went over it earlier, but the syntax is:

PIP <destination>=<source><pattern>

To copy PIP.COM into M: from the A: drive, you would type:

PIP M:=A:\PIP.COM

Wildcards can also be used.  To copy all *.COM files into your M: drive:

PIP M:=A:\*.COM

Now all of this is great, but we surely don’t want to type all this in every time we boot up the system.  Luckily, CP/M has a way of automatically executing a file called PROFILE.SUB each time you boot up, if it exists.  This is similar to AUTOEXEC.BAT under MS-DOS.

Place your Utility disk into your second drive.  Run ZDE.COM.  This is a full screen text editor that is easy to use.  Ensure your system disk is in drive A:

Like a MS-DOS batch file, this file contains commands that are executed as you would normally enter in at a CP/M command prompt.

Enter setdef m:,a:,* and press return

Hit ESC-S (top left-key on your keyboard) and give the file a name of A:PROFILE.SUB.  Hit ESC-X to exit.

The next time you boot up, your machine will automatically look in the M: drive first, then the A: drive, then whatever drive you’re currently in.

You can add as many lines as you’d like to the file.  To reopen your file, run ZDE.COM again, and use ESC-L to load your file.

Some other entries you can add to your PROFILE.SUB file:

Conf 40col=off - this makes the system faster by turning off 40 column mode

Conf back=x – change your background color to color x.

Conf char=x – change your foreground (text) to color x.

You can also list the contents of the PROFILE.SUB, as well as any text file, by using the TYPE.COM command.  From a command prompt:

TYPE.COM a:PROFILE.SUB

How to Make CP/M Even Easier

To make CP/M that much easier, the next step would be to get a CMD HD or compatible.  CMD has a native partition mode called 1581-C which allows you create a bootable CP/M image from your CMD HD.

We won’t go into how to create partitions here, however ensure when you do create one you are using the 1581-C format, and not the 1581 format.  The difference is where it writes its directory track.  CP/M has a different BAM layout than the 1581 and the 1581-C partition accounts for this.

You can create your system setup the same way as above, substituting the CMD HD for your second 1571.

The CMD HD can only use 1581-C partitions, which means it’s limited to an 800K partition.  However, you can have as many of these as you want.  Using a utility included called SPORT.COM, you can send Commodore DOS commands to the CMD HD from CP/M.  In order for you to be able to switch between your partitions, you’ll need to either have SPORT.COM on your system drive, or better yet, in your REU.  This allows you to call it up from partitions that may not have SPORT.COM copied to it.

In this example, we’re going to assume we have a CP/M partition at partition #18 that we want to switch to.

SPORT.COM’s usage is:

SPORT.COM /D<Device> /S<Secondary Address> /C<Command String>

It looks complicated, but it’s really not.  If I’m already on drive D:, and D: is my CMD HD, and I want to switch to partition #18, the command is:

SPORT /C’CP18’

And that’s it.  The command in the ticks is the standard CP (Change Partition) command that CMD HD DOS uses to change partitions.

How Do I Get Files to the CP/M environment?

Since it’s unlikely you’ll end up shopping for new CP/M software from your local store, the most often asked question is how to get files to the CP/M environment from Commodore DOS?

There’s a few ways of doing this.  The easiest way is to create a D64 on your PC/Mac/Linux box and copy the CP/M files you want to bring into your CP/M environment into the D64.  Again, there are so many ways to create D64s, I will not cover it in this guide.

Once your D64 is created, create a real disk from this D64.  From this point, you can:

Use Big Blue Reader 128 to bring files from this D64 in Commodore 128 mode into your CP/M environment as BBR 128 allows you to read and write CP/M disks.

Use RDCBM.COM to copy files in CP/M mode from Commodore DOS to CP/M

Use MSDOS.COM from CP/M mode to copy files to and from 360K MS-DOS disks.

Find a terminal program and download files from a BBS.

Summary

Hopefully you’ve found this a bit useful.  I’d like to expand this later with how to use a terminal program and such within CP/M as well as add some more useful information.  If you have any questions, etc., reach out to our group on Facebook at http://www.facebook.com/particlesbbs, or hit the BBS at http://www.particles.org

Brian Green 11/21/2015 v1.0